TERMS OF REFERENCE

for the purchase of a CIMC low-bed semi-trailer for Mine Operations Department, Kumtor Gold Company CJSC

| Item № | List of basic data and requirements | Basic data and requirements |
| --- | --- | --- |
| 1. | Delivery location | Issyk-Kul Region, Jeti-Oguz District, Kumtor Gold Mine |
| 2. | Client | Kumtor Gold Company CJSC |
| 3. | Subject of the purchase | Semi-trailer to transport pit equipment, wheeled and tracked machinery. |
| 4. | Purpose of the purchase | Replacement of obsolete equipment. |
| 5. | General requirements/ Regulations and standards | - The trailer shall be wheeled with a minimum load capacity of 85 tons for operation on the mine's pit roads. - Parameters of the trailer shall be as follows: - width of working platform shall be at least 3,800 mm, - length of working platform shall be at least 6,500 mm. - Towing hitch (Kingpin 2-3 1/2") for Volvo FMX 460 truck tractor-based towing. - Suspension with balanced axles is provided in Appendix 1. ІЅО10х335 - Trailer design is provided in Appendix 2. - All necessary documentation (certificates, passports, etc.) in accordance with the regulations and standards of the Kyrgyz Republic (EAEU) shall be available. |
| 6. | Individual requirements | - The semi-trailer shall be able to operate in high altitude conditions above 4,000 m above sea level, at the temperatures from - 40℃ to +40℃ (pneumatic braking systems shall be adapted to cold conditions). - Coupling and uncoupling design of the semi-trailer with the truck tractor shall ensure safe, quick, and reliable fixation with minimum human resource involvement. - The semi-trailer shall ensure safe and reliable fixation of equipment during transportation (fixation and external influence protection systems shall be available. - The semi-trailer shall be equipped with a balancing system to compensate for vibrations that occur during transportation on uneven surface. - The semi-trailer shall be equipped with a pneumatic braking system with the possibility of connecting a Volvo FMX 460 based tractor. - Tires 12.00R-20 shall have appropriate treads to operate in mountain conditions. |
| 7. | Payment and delivery terms | - 50% prepayment and 50% payment after commissioning. - The semi-trailer shall be delivered by the manufacturer to the BMY within the deadlines specified by the Client. - The semi-trailer shall be assembled and commissioned by the manufacturer within the deadlines specified by the Client. - In case of failure to comply with the terms, penalties shall apply. |
| 8. | Warranty period | - The Supplier shall provide a warranty period for the purchased equipment in accordance with the manufacturer's terms and conditions. |
| 9. | Client's services | - The manufacturer's employees performing assembling and debugging of the trailer at the mine site will be accommodated in the residential camp. - Internal transport. - Meals. - Additional services will be provided upon prior request of the Contractor for a separate fee. |
| 10. | Assembly, acceptance, transfer, and commissioning | - The Supplier shall complete assembly, adjustment, and commissioning work in full. - Acceptance Certificate shall be signed after the semi-trailer has been fully assembled and commissioned. - All costs for additional work identified during the equipment inspection shall be borne by the Supplier. |
| 11. | Training and operation manual | - The Supplier shall train personnel on maintenance and operation of the semi-trailer. - Operation manual for the semi-trailer (electronic, paper version) shall be provided (including in Russian language). - Spare parts catalogue (electronic, paper version) shall be provided. |
| 12. | Quality evaluation criteria | - After commissioning, the equipment shall be evaluated during the warranty period and shall comply with the manufacturer's stated specifications. - In the event of non-compliance with the requirements, the manufacturer shall bear all costs for its elimination. |
| 13. | Confidentiality | - The transaction data shall be confidential. The Contractor shall be liable for disclosure to third parties. |
| 14. | Environmental requirements | - All works shall be conducted in accordance with the KGC internal regulations. |
| 15. | Safety | - The semi-trailer shall comply with the safety requirements established by the applicable legislation of the Kyrgyz Republic. - All works shall be conducted in accordance with the KGC internal regulations. |

Appendix 1:

ЯЅ0 ВвЕ

DШ. А DN. В

oa.c

DN. С DЖ. D

ARGONAUT

Appendix 2: